Dahua CCTV Camera Integration

Falcon S1 Lite (NXP i.MX8MP) using GStreamer & RTSP

# 1. Overview

This document describes the steps taken to integrate a Dahua CCTV camera with the Falcon S1 Lite custom ARM64 board (NXP i.MX8MP) running Linux, using GStreamer and RTSP protocol for live video streaming.

| Item | Detail |
| --- | --- |
| Board | Falcon S1 Lite (Custom ARM64) |
| Processor | NXP i.MX8MP |
| OS | Linux (Ubuntu 22.04 / Yocto) |
| Camera | Dahua CCTV |
| Camera IP | 192.168.0.220 |
| RTSP Port | 554 |
| Protocol | RTSP over TCP |
| Streaming Tool | GStreamer 1.22.5 |
| Hardware Decoder | vpudec (NXP VPU) / v4l2h264dec |

# 2. Network Setup

## 2.1 Camera Network Configuration

The Dahua camera default IP is 192.168.1.108. A static IP was assigned through the Dahua web dashboard to place it on the same subnet as the board.

| Parameter | Value |
| --- | --- |
| Camera IP Address | 192.168.0.xx |
| Subnet Mask | 255.255.255.xx |
| Gateway | 192.168.0.1 |
| RTSP Port | 554 (default) |
| Username | Your username setup |

NOTE: The camera was also configured to connect via WiFi, so it is not dependent on a LAN cable after initial setup.

## 2.2 Verify Network Connectivity

Before running any GStreamer pipeline, verify the board can reach the camera:

# Test camera reachability

ping -c 4 your ip

# Check if RTSP port 554 is open

nmap -p 554 your ip

# or

nc -zv your ip 554

# 3. RTSP Stream URL

## 3.1 URL Format

rtsp://username:password@ip:port/cam/realmonitor?channel=1&subtype=0

| Parameter | Description |
| --- | --- |
| username | Camera login username |
| password | Camera login password |
| ip | Camera IP address |
| port | RTSP port (default 554) |
| channel | Camera channel number (starts from 1) |
| subtype=0 | Main stream - HD resolution (1280x720) |
| subtype=1 | Sub stream - low resolution (640x480) |

## 3.2 Test Stream in VLC Player

Before testing on the board, verify the stream works in VLC Media Player:

- Open VLC Player

- Go to Media > Open Network Stream

- Enter the RTSP URL above

- Click Play — video should appear

# 4. GStreamer Setup

## 4.1 Install GStreamer

sudo apt update

sudo apt install -y \

gstreamer1.0-tools \

gstreamer1.0-plugins-base \

gstreamer1.0-plugins-good \

gstreamer1.0-plugins-bad \

gstreamer1.0-plugins-ugly \

gstreamer1.0-rtsp

## 4.2 Verify Available Decoders

Check which H264 decoders are available on your board:

gst-inspect-1.0 | grep -i '264'

gst-inspect-1.0 | grep -i 'decode'

gst-inspect-1.0 | grep -i 'v4l2'

On the Falcon S1 Lite (i.MX8MP), the following hardware decoders are available:

- vpudec — NXP VPU hardware decoder (recommended)

- v4l2h264dec — V4L2 H264 hardware decoder (fallback)

## 4.3 Decoder Comparison

| Decoder | Type | CPU Usage | i.MX8MP Native | Status |
| --- | --- | --- | --- | --- |
| avdec_h264 | Software (FFmpeg) | High | No | BROKEN - version mismatch |
| v4l2h264dec | Hardware (V4L2) | Low | Partial | Working |
| vpudec | Hardware (NXP VPU) | Very Low | Yes | Recommended |

NOTE: avdec_h264 is broken on this system due to a GStreamer version mismatch (plugin v1.20.3 vs core v1.22.5). Always use vpudec or v4l2h264dec instead.

# 5. Working GStreamer Pipeline

## 5.1 Set Display Environment

Before running any pipeline, make sure the DISPLAY variable is set:

export DISPLAY=:0

NOTE: Without this, the pipeline will fail with 'Could not open display (null)' error.

## 5.2 Final Working Pipeline

Recommended pipeline using NXP VPU hardware decoder:

export DISPLAY=:0 && gst-launch-1.0 \

rtspsrc location="rtsp://username:password@ip:554/cam/realmonitor?channel=1&subtype=0" \

protocols=tcp latency=100 \

! rtph264depay ! h264parse ! vpudec ! videoconvert ! xvimagesink

Fallback pipeline using v4l2h264dec:

export DISPLAY=:0 && gst-launch-1.0 \

rtspsrc location="rtsp://username:password@ip:554/cam/realmonitor?channel=1&subtype=0" \

protocols=tcp latency=100 \

! rtph264depay ! h264parse ! v4l2h264dec ! videoconvert ! xvimagesink

## 5.3 Pipeline Explanation

| Element | Function |
| --- | --- |
| rtspsrc | Connects to the camera and receives the RTSP stream over TCP |
| rtph264depay | Extracts H264 video data from RTP packets |
| h264parse | Parses the H264 bitstream and prepares it for decoding |
| vpudec | Decodes H264 video using NXP VPU hardware (saves CPU) |
| videoconvert | Converts video format for display compatibility |
| xvimagesink | Renders the decoded video to the X11 display window |

# 6. Run Script Setup

## 6.1 Create the Script

nano ~/run_dahuacctv.sh

Paste the following content:

#!/bin/bash

export DISPLAY=:0

export XDG_RUNTIME_DIR=/run/user/$(id -u)

RTSP_URL="rtsp://username:password@ip:554/cam/realmonitor?channel=1&subtype=0"

# NXP VPU hardware decoder - best for i.MX8MP

gst-launch-1.0 \

rtspsrc location="$RTSP_URL" protocols=tcp latency=100 \

! rtph264depay ! h264parse ! vpudec ! videoconvert ! xvimagesink

## 6.2 Make It Executable

chmod +x ~/run_dahuacctv.sh

## 6.3 Run the Script

~/run_dahuacctv.sh

## 6.4 Add Permanent DISPLAY Variable

To avoid manually setting DISPLAY after every reboot:

echo 'export DISPLAY=:0' >> ~/.bashrc

echo 'export XDG_RUNTIME_DIR=/run/user/$(id -u)' >> ~/.bashrc

source ~/.bashrc

# 7. Auto-Start on Boot (Optional)

This section is only needed if you want the camera stream to start automatically on every boot without manual intervention. This is recommended for final product deployment.

## 7.1 Create systemd Service

sudo nano /etc/systemd/system/camera-stream.service

Paste the following content (replace 'user' and '1000' with your actual username and user ID):

[Unit]

Description=Dahua Camera RTSP Stream

After=network.target graphical.target

[Service]

Type=simple

User=user

Environment=DISPLAY=:0

Environment=XDG_RUNTIME_DIR=/run/user/1000

ExecStart=/home/user/run_dahuacctv.sh

Restart=always

RestartSec=5

[Install]

WantedBy=graphical.target

NOTE: Run 'whoami' to get your username and 'id -u' to get your user ID number.

## 7.2 Enable and Start the Service

sudo systemctl daemon-reload

sudo systemctl enable camera-stream.service

sudo systemctl start camera-stream.service

sudo systemctl status camera-stream.service

## 7.3 Useful Service Commands

| Command | Description |
| --- | --- |
| sudo systemctl status camera-stream.service | Check if stream is running |
| sudo systemctl start camera-stream.service | Start the stream manually |
| sudo systemctl stop camera-stream.service | Stop the stream |
| sudo systemctl restart camera-stream.service | Restart the stream |
| journalctl -u camera-stream.service -f | View live logs |

# 8. Troubleshooting

| Error / Symptom | Cause | Fix |
| --- | --- | --- |
| Could not open display (null) | DISPLAY variable not set | Run: export DISPLAY=:0 |
| No such element 'avdec_h264' | GStreamer version mismatch (1.20.3 vs 1.22.5) | Use vpudec or v4l2h264dec instead |
| Failed to connect (Generic error) | Camera unreachable on network | Ping camera: ping ip |
| wl_display error: invalid method | Wayland protocol version mismatch | Use xvimagesink instead of waylandsink |
| libgstwaylandsink.so: undefined symbol | Broken Wayland plugin | Run: sudo apt install --reinstall gstreamer1.0-plugins-bad |
| Pipeline runs but no video | Wrong decoder output format | Switch between vpudec and v4l2h264dec |
| Stream lag/buffering | High network latency | Reduce latency=100 or use subtype=1 |

# 9. Quick Reference

## After Every Reboot — Run This:

~/run_dahuacctv.sh

If the script does not start (display error):

export DISPLAY=:0 && ~/run_dahuacctv.sh

## Pipeline at a Glance:

rtspsrc → rtph264depay → h264parse → vpudec → videoconvert → xvimagesink